Ad-Hoc BPL Working Group: Verbal Report

C63 meeting, April 28, 2005, Piscatway, NJ -- Prepared by Ed Hare, ARRL

FCC Rulemaking  (historical) http://hraunfoss.fcc.gov/edocs_public/attachmatch/FCC-04-245A1.pdf

- Did not change emissions limits – Below 30 MHz, rules would permit 29.54 dBuV/m at 30 meters distance (69.54 dBuV/m at 3 meters distance, 48.62 dBuV/m at 10 meters distance)

- Did not change requirement not to cause harmful interference

- Commissioners statements stressed both benefits of BPL and importance of addressing interference

- List of 12 prohibited frequency bands

- 20-dB suppression in prohibited bands is adequate on HF, 10 dB on VHF

- Extensive list of geographical exclusion zones

- Clarifies measurement methods for access BPL

  - Use loop antenna at 1-meter height – Above 30 MHz, 5-dB height correction or making multiple measurements at different heights

  - 10-meters horizontal separation – continue 40-dB/distance decade extrapolation for line radiators

  - Measure up and down the line to find maximum, from 0 to 1 wavelength in ¼ wavelength instruments

  - Measure 3 typical overhead and 3 typical systems in situ

  - Measure at compass points around ground-mounted transformers and houses

  - Some additional measurements required for in-home BPL

- Requirement for design to be adjustable in frequency, power level and remote shutoff

- Certification instead of Verification

- Must test with device set to maximum power and report power level that achieved compliancePublic database of zip code, frequency and contact information

- FCC will create unspecified step-by-step procedure for identifying and correcting interference

- Equipment authorized under existing rules can be sold under transitional rules for 18 months

- Above 30 MHz, comply with Class A limits for emissions from overhead power lines

- Access BPL exempt from 15.107( c ) conducted emissions limits

- Definition of access BPL

Rules clarify measurement methods for access BPL

- Use loop antenna at 1-meter height – Above 30 MHz, 5 dB height correction or making multiple measurements at different heights

- 10-meters horizontal separation – 40 dB/distance decade extrapolation for line radiators

- Measure up and down the line to find maximum, from 0 to 1 wavelength in ¼ wavelength instruments

- Measure 3 typical overhead and 3 typical systems in situ

- Measure at compass points around ground-mounted transformers and houses

- Some additional measurements required for in-home BPL

- Additional industry standards and specifications work in progress (IEEE, Electric Power Research Institute)

Petitions for Reconsideration

- ARRL challenging on technical and legal grounds

- ARRL claims FCC procedural errors

- FCC testing was used in Report and Order, but never released. 650 pages obtained under FOIA

- ARRL claims this testing has results inconsistent with Report and Order

- ARRL asking that 40 dB/distance decade extrapolation not be used for line emitters below 30 MHz

- Industry asking that public database information not be required prior to operation

- Industry asking to extend 18-month grandfathering for certification

From FCC Test Data Released Under FOIA

Redacted Material

IEEE Standards Association formed Study Group

- Out of this, 3 major types of standards under development

- Safety and installations standards being done by PES (P1675)

- EMC standards being done by PES, with EMCS as a co-sponsor (P1775)

- Hardware and protocol standards being done by PES, with the Communications Society as co-sponsor

- There were considerable difficulties the formation of the EMC-standards group

- End result was that PES will sponsor, with EMCS as co-sponsor

- Aron Viner, Ambient (BPL manufacturer) chairs and for now, Steve Berger is co-chair

- EMC group working on test methods and procedures

- Intentional decision not to address compatibility with radio services or to set specific emissions limits in standard

- http://grouper.ieee.org/groups/bpl

Other Standards Work

- C63

- EMC Society Standards Development Committee study project

- Will be ramping up now that C63 meeting over

- Will probably encompass the broader topic of the EMC aspects of broadband distributed emitters

- Internationally, CISPR essentially starting from scratch

- German, Japan and Australia all also looking at regulation and standards changes

- BPL Track at EMC Symposium in Chicago

Field Deployment

- In general, field deployment increasing

- Major commercial deployment in Cincinnati area

- Operated by Cinergy, using Current Technologies modems

- HomePlug notching – interference potential reduced

- Other systems with active interference complaints

- FCC has not been aggressive with respect to enforcement

- Systems in Cedar Rapids, IA; Raleigh, NC; PennYan, NY; Irving, TX and others have shut down

- ARRL working with some vendors and operators to identify and resolve interference, to varying degrees of success and lack thereof

- Some are addressing interference, some are not and some are only partially addressing interference

EMC issues

- Some have formed basis of petitions for recon

- Proper test methods

- Extrapolation vs height (0 dB below 30 MHz, +5 dB on VHF and above)

- Extrapolation vs distance (40 dB/decade below 30 MHz,  20 dB/decade on VHF and above)

- Compatibility with radio services

- Other?